Operator _______________________	Laptop _______________________		Date _______________________	     Mooring ______________________

| S/N | TEST 1. Online Set clock 2. Online Set pressure offset 3. Start range check* Stop data recording | SETUP Measurement int = 3600s Avg interval = 60 s Measurement load = auto Blanking dist = 0.5 m Compass upd rate = 1 s | SPEED OF SOUND fixed @ 1500 m/s | COORD SYS ENU | DIAGNOSTICS Enabled Interval = 720 min No. samples = 20 | CHECK DEPLOY PLANNING** | DEPLOY 1. start deployment recorder 2. start date 3. start time |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

* Check that the amplitude from the 3 beams is similar and ~ 30-50m across the range.

** Select batt pack = 2*Alkaline  / capacity 100Wh

Assumed duration = 730 days

Should read:

Battery utilization = 79%

Vertical vel prec = 1.4 cm/s

Horizontal vel prec = 0.9 cm/s

Operator _______________________	Laptop _______________________		Date _______________________	     Mooring ______________________

| S/N | TEST 1. Online Set clock 2. Online Set pressure offset 3. Start range check* Stop data recording | SETUP Measurement int = 3600s Avg interval = 60 s Measurement load = auto Blanking dist = 0.5 m Compass upd rate = 1 s | SPEED OF SOUND fixed @ 1500 m/s | COORD SYS ENU | DIAGNOSTICS Enabled Interval = 720 min No. samples = 20 | CHECK DEPLOY PLANNING** | DEPLOY 1. start deployment recorder 2. start date 3. start time |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

* Check that the amplitude from the 3 beams is similar and ~ 30-50m across the range.

** Select batt pack = 2*Alkaline  / capacity 100Wh

Assumed duration = 730 days

Should read:

Battery utilization = 79%

Vertical vel prec = 1.4 cm/s

Horizontal vel prec = 0.9 cm/s