# Signature 55Hz – Postprocessing guidelines

Link to this guide:

Signature 55Hz processing guide.docx

### Last update: 08/05/2026 16:58:08

- RAW-file: use for post-processing

- Averaged file: Quick check for onboard (plot STAGE 1 check list)

- Current git branch:

https://github.com/ScotMarPhys/m_moorproc_toolbox/tree/dy181_RHADCP

- Other matlab toolboxes needed: rodb, GSW v3.06

## QC flags:

- 0: QC_NOT_EVALUATED

- 1: QC_GOOD

- 2: QC_UNKNOWN

- 3: QC_PROBABLY_BAD

- 4: QC_BAD

- 5: QC_CHANGED

- 6: QC_UNSAMPLED

- 7: QC_INTERPOLATED

- 8: QC_?

- 9: QC_MISSING

## STAGE 0 – Download data

Download raw data (*.ad2cp and *avgd.ad2cp) from instrument using the Recorder Data Retrieval tab in Nortek Signature Deployment software. Open ad2cp-files in Signature Viewer and export to mat-files (*.mat, *avgd.mat). This will automatically apply a quality control with only data where the correlation of echo and signal is above 50% is kept in the avgd.mat file.

Please ensure to rename the files in line with OSNAP processing toolbox:

Naming conventions: [serial_number]_data.mat, [seriel_number]_data_avgd.mat

E.g. RHADCP has serial number 20044: 20044_data.mat; 20044_data_avgd.mat

Copy to folder: \osnap\data\moor\raw\cruise_id\adcp_s55\

E.g.: osnap\data\moor\raw\jc238\adcp_s55\

If not yet done for the microcat, create a data processing control file [moorname]info.dat for the mooring in the directory osnap/data/moor/proc/[moorname]. [moorname]info.dat contains metadata for mooring position (lon, lat, waterdepth and mean magnetic deviation during the deployment), deployment period, nominal depths and serial numbers of each instrument.

Example for JC238, mooring deployment #1 for RHADCP from 2020-2022:

moorname = rhadcp_01_2020

Example: osnap\data\moor\proc\rhadcp_01_2020\rhadcp_01_2020info.dat

Mooring              = rhadcp1_01_2020

WaterDepth           = 1080

MagDeviation         = -7.07

Start_Date           = 2020/10/15

Start_Time           = 19:30

End_Date             = 2022/07/20

End_Time             = 09:30

Latitude             = 57 36.89 N

Longitude            = 15 24.66 W

Columns              = z:instrument:serialnumber

1068	324	200044

1070	337	9378

## STAGE 1 – General Quality Control Checks (Nortek)

Wrapper: m_moorproc_toolbox\exec\gitrepo\stage1\adcp_s55\call_stage01_read_qc_S55_JC238.m

function: m_moorproc_toolbox\exec\gitrepo\stage1\adcp_s55\stage01_read_qc_S55.m

Should work with and without toolbox global variables set. Needed matlab toolboxes:

- m_moorproc_toolbox\exec\gitrepo\ needs to be added to matlab path for rodbload.m

- GSW v3.06 needs to be added to matlab path

All checks should use plots showing original and flagged data. ONLY check and flag, processing is stage 2:

### Figure 1 – status: done

- Pressure: Must be reasonable; shallow readings may indicate deployment/retrieval and invalidate profiles.

- Instrument Orientation:

  - Tilt <10°: good

  - Tilt 10–30°: post-processing possible

  - Tilt >30°: profiles likely erroneous (one transducer horizontal, velocity component undetectable)

- Tilt & Heading: Should remain fairly constant during deployment.

### Figure 2 and Fig 3 – status: done (point 6 pending)

- Signal Level per Beam and Correlation:

  - Should start high, decrease exponentially with range.

  - Sudden increase = surface boundary; affected cells likely corrupted.

  - If no boundary, signal stabilizes; valid if above noise floor.

  - Cells near noise floor are invalid; measurements beyond this range are erroneous.

  - Do we have a minimal signal strength?

- Single Bad Beam: If one beam is near noise floor throughout would we need to discard all data as we only have 3 beams to start with?

- Spikes in Signal Strength: Flag and remove affected cells; develop auto-processing algorithm.

- Caused by frame, mooring parts: appears as constant stripe in amplitude

- Fish, etc.:

- GO-SHIP vmADCP: three-profile running median of the received amplitude for each beam – not sure how to apply that post processing

- I calc median and standard deviation of 10samples/10second ensembles: if larger then median plus 3*standard deviation (~ 0.05% of data) then flag.

- Along depth dimension I calc diff of ensemble median, then flag any diff larger then median+/-3*std of diff and remap to oridinal size (0% data). I do not apply these flags at the moment. Just print out a suggestion to smooth/despike velocity

- Just check again Correlation (<50 will be set to NaN by default)

- Check side lobe interference:

As RHADCP is mounted on string I applied the following including tilt β. This is calc as:

Rmax=H cosβ-Cell Size

Rmax:	maximum range without sidelobe interference

H:	instrument depth (max out of user set nominal depth and temporal mean based on pressure sensor

β:	time varying maximum tilt of three beams compared to vertical

### Figure 4 – status: done

- Unrealistic Current Variations: >1 m/s over few meters or velocities far from mean should be removed.

- Vertical Velocity Check: Should be near zero. Cells with implausible vertical velocities (>0.1m/s) should be discarded.

### Figure 5 – status: WIP

- Compass Offset: Plot horizontal components as a circle.

  - Offset = “Hard Iron” rotating with instrument. See blue dots in figure 20.

  - Ellipse = “Soft Iron” nearby.

  - ONLY check and flag, processing is stage 2

Using the magnetometer data:

- Raw Measurement: The magnetometer outputs  My, Mx which contain the Earth's magnetic field distorted by the instrument frame's Hard-Iron error. The raw magnetic heading is calculated as: ψraw=atan2(-My, Mx)

- Calibration: By fitting a circle to the raw My, Mx data points, we identify the offsets yc,xc. The corrected heading relative to Magnetic North is found by centering the data: ψtrue=atan2-(My-yc), Mx-xc

- Correction Angle: The angular deviation caused by the Hard-Iron effect is:

ϵ=ψtrue-ψraw

- Velocity Transformation: To align the measured velocities Uraw, Vraw with the corrected heading, we apply a rotation matrix using the error ϵ:

Ucorr=Urawcosϵ-Vrawsinϵ

Vcorr=Urawsinϵ+Vrawcos⁡(ϵ)

The function s55_hard_iron_compass_correction.m calculates the center of the compass data (hard iron). Here we only plot it, but is has the functionality to correct U and V accordingly, which can be used in stage 2.

### Figure 6 – status: completed

Plot all sorts of sensor diagnostics (temperature sensors, accelerometers, magnetometer). Just check then temperature sensors are sensible, etc.

### Save Stage 01 data

Data are saved filename: moor_id_sn_stage1.nc,  e.g. for JC238: rhadcp_01_2020_200044_stage1.nc

Data directory:  osnap/data/moor/proc/moor_id/adcp_S55/

E.g. for JC238:  osnap\data\moor\proc\rhadcp_01_2020\adcp_S55\

The function s55_load_nc_as_struct.m loads nc-file as matlab structure.

## STAGE 2 – Post-processing

Status: WIP

Wrapper: m_moorproc_toolbox\exec\gitrepo\stage2\adcp_s55\wrapper_stage02_proc_s55.m

Function: m_moorproc_toolbox\exec\gitrepo\stage2\adcp_s55\stage02_proc_s55.m

- Calculate mid depth of each bin: depth from pressure records corrected by distance of first bin to transducer depth(, depth offset) and speed of sound mean profile

  - In post-processing check pressure sensor drift by using calibrated MC pressure sensor data

  - Use also MC measurements for speed of sound correction.

- Check side lobe interference:

As RHADCP is mounted on string I applied the following including pitch.

Rmax=H cosθ0+θpitch-Cell Size

Is this automatically flagged as NaN by the processing software? Do we supply a guess of surface distance (hcorr) in the instrument settings?

I recall that in practice, contamination may extend beyond this limit due to surface roughness.

Would the side lobe interference be detected by the correlation between echo and signal (e.g., dropping below 50%)?

- Hard iron compass correction

- Correct horizontal velocity for Magnetic Deviation

## STAGE 3 – Delayed time post-processing

Run stage 2 again with calibrated MC data

Stage 3 – grid on finer z grid?

## Appendix

Other notes just to have some equations:

Calculate depth from surface signal in beam amplitude:

- Quadratic fit over surface bins to determine bin fraction with highest echo amplitude defining the surface

- Calculate nominal bin distances to surface (bin-1)*cell_size - (i.e. distance of first bin to first bin is 0, second bin is 1*cell_size, etc) - interpolate nominal bin distances on surface bin fraction - This give you the distance of the first bin to surface (uncorrected) hi (for each beam)

- Correct for speed of sound mean climatology profile at ADCP location (ceff: harmonic mean over all depth - use pressure sensor here, cinstr: set to 1500m/s):

hic=ceff/cinstr*hi

- Then they add the distance from first bin to transducer

- From that the correct for the beam width, accounting for transmission loss:

  - EL=SL-2αR-20log10(R)

  - What is absorbtion coeff α for 55kHz?

  - What is the beam slant angle for 55kHz? - 20°?

  - Surface roughness parameter ( aether2 ) is computed from wind speed at the mooring:

aether2=0.003+5.12×10-3×wspadcp

- For each time step:

Ss=-10*log8 π aether2-2.17aether2 tan(90-ϴ)360 2 π2

- The reflected echo level is: EL = SL + TL + S_s

- The beam angle with maximum echo level defines the effective angle:

θeff = mean(θmaxEL)

- For each beam ( i ) and time step:

hib=cosθeff360 2πcosθ0360 2πhi	, θ0 = beam slant angle - 20° for S55?

- Correct for pitch and roll - need new code for 3 beam solution and measured pitch and roll:

θeff =arccoscosθ0-sinϕp*sinθ0*cosψi+sinϕr*sinθ0*sinψi

hcorr= hib*cosθeffcosθ0

Where:

θ0= nominal beam angle

ϕp= pitch rotation

ϕr= roll rotation

ϕi= azimuth angle of the beam (e.g., 0°, 120°, 240° for beams 1–3)

- Estimate depth offset: compare mean depth derived from pressure sensor with mean depth derive from surface amplitude